RHEM-Snow Technical Documentation

Patrick Broxton

University of Arizona

broxtopd@arizona.edu

April 2021

Contents:

- Introduction ------------------------------------------------------------------------------------------	1

- Model Process Description -----------------------------------------------------------------------	2

2.1)   Precipitation and Humidity ---------------------------------------------------------------	2

2.2)   Solar Radiation -------------------------------------------------------------------------------	3

2.3)   Longwave Radiation ------------------------------------------------------------------------	4

2.4)   Net Radiation and Albedo -----------------------------------------------------------------	4

2.5)   Sensible and Latent Heat -----------------------------------------------------------------	5

2.6)   Heat from Precipitation -------------------------------------------------------------------	7

2.7)   Ground Heat ----------------------------------------------------------------------------------	7

2.8)   Melt and Snowpack Mass and Energy Balance --------------------------------------	7

2.9)   Canopy Interception ------------------------------------------------------------------------8

2.10)   Snow Density -------------------------------------------------------------------------------	9

2.11) Near Surface Hydrology Model ----------------------------------------------------------	10

- Forcing Data ------------------------------------------------------------------------------------------	12

- Model Parameters ----------------------------------------------------------------------------------	13

- Running RHEM-Snow ------------------------------------------------------------------------------	16

5.1)   Input Data -------------------------------------------------------------------------------------	16

5.2)   Model Execution ----------------------------------------------------------------------------	17

5.3)   Model Output --------------------------------------------------------------------------------	17

- Explanation of Model Codes ---------------------------------------------------------------------	18

- References -------------------------------------------------------------------------------------------	20

1) Introduction

RHEM-snow is a computationally efficient snow model that is designed for use with the USDA’s RHEM / Kineros2 (K2) models with daily Cligen forcing data.  It is a moderately complex model, having treatments for the snowpack mass and energy balance, but was designed to be computationally efficient with low data requirements.  This makes it ideal for running the model over large sets of stations, or in applications where nearly instantaneous results are required.  RHEM-Snow is designed to be run with the standard set of forcing inputs produced by the CLIGEN weather generator (daily inputs of precipitation, maximum and minimum temperature, dewpoint temperature, solar radiation, and wind speed), but can also be run using other similar sets of daily forcing data, optionally, including longwave radiation as well as precipitation inputs that are already partitioned between rainfall and snowfall.  The codes are available in Python.

2) Model Process Description

RHEM-Snow calculates snowmelt and sublimation using a hybrid approach between simple degree-day modelling, and more complex energy balance modelling.  It calculates all snowpack mass and energy balance terms.  However, due to the desired computational constraints of the model (computational efficiency and daily forcing information), computation of energy balance is somewhat simplified (i.e. the snow surface temperature is an empirical function of air temperature, humidity, and net incoming radiation).  The model also estimates changes in snow density, and optionally, canopy interception. Furthermore, it also includes formulations to compute necessary adjustments to forcing data if needed: partitioning between rainfall and snowfall, computation of incoming longwave radiation, and adjustment of solar radiation on inclined surfaces. There is also a simple model for keeping track of the temperature / moisture in the top soil layer.  Below is a comprehensive set of model formulations for various processes in RHEM-Snow.

2.1) Precipitation and Humidity

RHEM-snow can either incorporate already partitioned rainfall and snowfall, or it has algorithms to separate the rainfall and snowfall based on air temperature.  If required, rainfall-snowfall partitioning in RHEM-Snow is accomplished using a temperature range, the lower limit of which is all snowfall, and the upper limit of which is all rainfall.  We use a continuous function to model this transition:

fs=max⁡0,min⁡1,1-dhdxTa-Ta,min-dh2πsin2πdxTa-Ta,min     (1)

In this expression, dh=1, dx=Ta,max-Ta,min, Ta,max=Ta+Ta,range/2, and Ta,min=Ta-Ta,range/2. T, the air temperature threshold, and Ta,range, the range over which the rain-snow transition occurs, are user defined parameters.

In addition to the rain-snow transition, there are also expressions to compute vapor pressure, saturated vapor pressure, and ultimately relative humidity (the ratio between the two).

Vapor pressure,  e, is computed from the daily dewpoint temperature, Td, using Teten’s formula (Monteith and Unsworth, 2008):

e=0.6108×exp⁡17.27×Td237.3+ Td×1000     (2)

Similarly, saturated vapor pressure, esat, is computed from the air temperature, T:

esat=0.6108×exp⁡17.27×Ta237.3+ Ta×1000     (3)

2.2) Solar Radiation

Solar Radiation is given as a forcing input.  However, RHEM-Snow also computes potential solar radiation for a particular location, including on inclined surfaces.  This potential radiation is used in two cases.  First, if longwave radiation is computed, the ratio of given to potential flat-surface solar radiation is used to estimate a cloud fraction, which affects the computation of incoming longwave radiation (see ‘Longwave Radiation’ section below).  Second, if shortwave radiation is adjusted for slope and aspect influences, given solar radiation values are multiplied by the solar forcing index computed for the hillslope, where

SFI=RsR0 	     (4)

Rs is the potential solar radiation on the inclined surface and R0 is the flat surface potential solar radiation.

The potential solar radiation (both R0and Rs) is computed using a computer code developed by Dr. Felix Hebeler, dept. of Geography, University of Zurich, slightly modified to accept the slope-aspect conventions in RHEM-Snow, and to output daily solar radiation values (and converted from Matlab to Python).  The code follows the approach of Kumar et al. 1997 (note that specific equations related to the algorithm can be found in that reference). It calculates clear sky radiation corrected for the incident angle (self-shading) plus diffuse and reflected radiation.  Seasonal timeseries of R0 also further adjusted to ensure a match between observed and modeled clear sky conditions by multiplying R0 by a seasonally varying correction factor, Tti, where

Tti=f*Rsummer+1-f*Rwinter     (5)

In this formula,

f=R0-min⁡(R0)maxR0-min⁡(R0)      (6)

and Rsummer and Rwinter are the ratios between the given and potential solar radiation (R0) during the summer and winter months, respectively.  Rsummer and Rwinter are found by dividing maximum observed solar radiation values with the potential flat surface solar radiation values for ~1 month periods centered on the summer and winter solstices.

There is also a user-adjustable parameter that applies a simple multiplier to the shortwave radiation (after any adjustments are made for slope and aspect) so the final downward shortwave radiation is computed is:

Rsd= Robs×SFI×fr     (7)

where Robs is the observed solar radiation, SFI, is the solar forcing index (from 1) and fr is the user-defined multiplier.

2.3) Longwave Radiation

If specified, incoming longwave radiation is computed using the Stefan-Boltzmann equation:

Rld=εaσTa4 	     (8)

where σ is the Stefan-Boltzmann constant (5.67×10-8Wm2K), Ta is the air temperature (in Kelvin), and εa is the effective emmisivity from the sky.  Emissivity is calculated as:

εa=CF+1-CF×εacls      (9)

where CF is the cloud fraction, and εacls is the clear sky emmisivity.  Parameterization of clear sky emmisivity follows Satterlund (1979):

εacls=1.081-exp-ea100Ta2016      (10)

where ea is the vapor pressure in Pa.  Here, the cloud cover fraction is estimated from the Bristow and Campbell (1984) transmission factor;

CF0=1-TFa      (11)

where the transmission factor, TF, is calculated here as the ratio between observed and calculated clear sky solar radiation for the modeled location.  a is a user defined parameter.  Cloud fraction is also limited such that a precipitation value over a certain threshold (right now set to 1 inch = 25.4 mm) always results in a cloudy day:

CF=maxmin1,P/25.4, CF0      (12)

where P is the daily precipitation amount in mm.  This is done to reduce the impact of a mismatch between dewpoint temperature and precipitation (days with lots of precipitation can occur on days where the humidity is pretty low in the Cligen data).

Like with shortwave radiation, there is a user defined parameter that applies a simple multiplier to the incoming longwave radiation (either measured or calculated).

2.4) Net Radiation and Albedo

Net Radiation is computed as:

Rn=Rsd1-α+Rld-Rlu     (13)

where Rsd is the incoming solar radiation, Rld is the incoming longwave, Rlu is the outgoing longwave radiation, and α is the snow albedo.

Rlu is calculated using the Stephan-boltzmann equation:

Rlu=εsσTs4      (14)

where εs is the snow emissivity (0.99), σ is the Stefan-Boltzmann constant (5.67×10-8Wm2K), and Ts is the snowpack temperature.  Due to the computational constraints of the model, rather than iteratively solving the energy balance, surface snow temperature is estimated using an empirical equation based on air temperature, relative humidity and net incoming radiation, fitted to field data from a field site in Arizona which has detailed measurements of radiation components and surface temperature:

Ts=Ta+0.0192×Qsn-Qli+0.648×RH-14.5601       (15)

where Ts is the snow surface temperature, Ta is the air temperature, Qsn is net (incoming-outgoing) solar radiation, Qli is incoming longwave radiation, and RH is the relative humidity (in percent).  Note that at the site where it was developed, this equation explains daily variation in Ts with an R2 of 0.78 and an RMSE of 0.86oC.

Snow albedo, α, is computed as a function of the age of the snow surface (since the last significant snowfall):

α=αi-αmin exp-τk+αmin      (16)

αi  is the albedo of fresh snowfall, αmin is the albedo of old snow, τ is the age of the snow surface, and k is a decay constant.  Every day, τ increments by one day, but on days that it snows, τ is adjusted (to 0 if the storm is large enough) using:

τ= τ0max⁡0,ar-Sar      (17)

Where S is the snowfall, ar is a parameter specifying the snowfall size that will completely reset the snow surface albedo.  αi, αmin, k, and ar are user defined parameters.

2.5) Sensible and Latent heat

Sensible heat flux when the temperature is computed as:

Qh=KhρacpTs-Ta    (18)

Here,  cp is the air specific heat capacity (1005 Jkg°K), Kh is the sensible heat conductance, ρa is the air density, Ta is the air temperature, and Ts is the snow temperature.

The variation of ρa, the air density, with elevation is estimated because air pressure is not given.  ρa is calculated as:

ρa=PRdTa        (19)

Where pressure is estimated to be:

P=P0×exp⁡-gzRdT0       (20)

Here, Rd, the specific gas constant for dry air, is 287.058 Jkg°K,  g, the acceleration due to gravity, is 9.81 ms2, and T0=Ta+L×z, L is an estimated lapse rate (6.5 °Kkm), P0 is standard sea level pressure (101.325 kPa) and z is the elevation.

The latent heat flux is computed as:

Qe=Ke0.622×hvRd×Taea-es(Ts)     (21)

where Ke is the latent heat conductance, es(Ts) is the saturated vapor pressure at the snow surface, ea is the air vapor pressure, Rd is the dry gas constant, hv is the latent heat of sublimation (2834 kJ/kg).

The water equivalence depth of sublimation is:

E=Qe×dtρwhv     (22)

ρw is the density of water (1000 kg/m3), and dt is the model timestep.

The heat conductance terms are given by expressions of turbulent transfer in the boundary layer (Anderson, 1976; Male and Gray, 1981; Brutsaert, 1982).  For neutral conditions:

Kneutral=Kh=Ke=k2Vln⁡zz02     (23)

Here, k is von karman’s constant (0.4), V is the wind speed at measurement height, z, and z0 is roughness height at which the logarithmic boundary layer profile predicts zero velocity.  Both z and z0 can be specified as model parameters.  A stability correction is applied based on the Richardson number, which is approximated as (ref):

Ri≈gTa-TszV2Ta       (24)

Ri is used to adjust Kh and Ke following Price and Dunne (1976):

Kadj=Kneutral1+10Ri, Ri≥0, Stable Conditions     (25)

Kadj=Kneutral1+10Ri, Ri<0, Unstable Conditions  (26)

As in Tarboton and Luce, 1996, there is also a stability factor to control the extent that these correction factors are applied:

Kh=Ke=Kneutral+FstabKadj-Kneutral    (27)

Fstab is a user defined parameter ranging from 0-1 (0 uses neutral conductances and 1 gives full stability corrections).

2.6) Heat from Precipitation

Heat from precipitation considers heat from rainfall and snowfall separately (assuming that rainfall temperature is the maximum of air temperature and freezing temperature, and snowfall temperature is the minimum of air temperature and freezing temperature):

Qp=PsCsρwminTa,0+Prhfρw+CwρwmaxTa,0     (28)

Ps is the snow rate, Pr is the rain rate, Cs is the specific heat of ice (2.05 kJkg-K), Cw is the specific heat of water (4.181 kJkg-K), ρw is the density of water (1000kgm3), and hf is the latent heat of fusion (334 kJkg-K).

2.7) Ground Heat

Ground heat flux is given by:

Qg=-kdTdz    (29)

k is the soil thermal conductivity (which is a user definable model parameter at this point, in the absence of information about soil type or wetness), dz is the distance between the damping depth (also a user definable parameter) and the middle of the snowpack, and dT is the temperature difference between the damping depth (also a user definable parameter) and the middle of the snowpack.

2.8) Melt and Snowpack Mass and Energy Balance

RHEM-Snow computes potential melt energy based on energy contributions of other energy balance terms:

Qm×dt=-max⁡(0,cc+Qn+Qh+Qe+Qp+Qg×dt)    (30)

Here, Qn is the net radiation, Qh is the sensible heat, Qe is the latent heat, Qp is the precip heat, Qg is the ground heat, dt is the model timestep, and cc is the cold content.

In this expression, SWE is the snow water equivalent, ρw is the density of water (1000kgm3), Ci is the specific heat of ice (2050 Jkg-K), and Tp is the lesser of the average of the last n days temperatures and 0oC (where n is a user-defined parameter).

Melt mass flux is computed as:

M=-Qm×dtρw×hf       (31)

Melt is limited such that it cannot exceed SWE (note that the energy flux is also limited).

RHEM-Snow does not account for the liquid holding capacity of water, so this melt is immediately removed from the snowpack.  The resulting mass balance of the snowpack is:

dSWEdt=Ps+Pr-E-M      (32)

where Ps is snowfall (which includes both throughfall and unloaded snow, if under canopy), Pr is rainfall (which includes both rainfall and drip from the canopy), E is sublimation, and M is snowmelt.

The energy balance is:

dccdt=Qn+Qh+Qe+Qg+Qm      (33)

Where cold content is limited so that the bulk snowpack temperature is between the snow surface and 0oC.  Note that if there is an imbalance, then excess heat is added to the sensible heating term.

2.9) Canopy Interception

Canopy interception is calculated from:

It=It-1+0.7Imax-It-11-exp⁡SImax-u-e-d   (34)

where It is the intercepted snow for the current timestep, It-1 is the intercepted snow for the previous timestep, S is the snowfall, Imax is the maximum interception capacity, given by u is the snow unloading rate, e is the canopy sublimation rate, and d is the melt-drip rate.  The maximum interception capacity is given by (Hedstrom and Pomeroy 1998):

Imax=4.4LAI    (35)

where LAI is the total leaf area index (that includes stems, leaves, and branches).  The term for new snow interception (0.7Imax-It-11-exp⁡SImax) is based on formulations from Liston and Elder, 2006 and Pomeroy et al., 1998. The amount of snow unloading  (u) is estimated as a function of the amount of intercepted snow:

u=pu×I   (36)

where I is intercepted snowfall (after the addition of new snowfall, but before subtractions from sublimation and drip), and pu is a user definable parameter specifying snow unloading rate.  The amount of sublimation of intercepted snowfall (e) is tied to calculation of sublimation from the snowpack by:

e=pe×e0      (37)

where e0 is snowpack sublimation, and pe is a user-defined multiplier representing how much faster canopy sublimation occurs relative to snowpack sublimation.  Finally, the melt drip rate is given by a temperature index formulation:

d=max⁡(0,pd×T)      (38)

where T is the temperature (in celcius), and pd is a user defined parameter representing the melt drip rate.

2.10) Snow Density

Snow density in RHEM-Snow is given by:

ρt=max⁡ρi,min⁡ρmax,1-f×ρt-1+f×ρi+ds+ds+dl          (39)

Here, ρt is the snow density for the current timestep, ρt-1 is the snow density for the previous timestep, ρi is a user-definable parameter representing the density of new snowfall,  ρmax is the maximum allowable snow density, f is the fraction of pack that is contributed by new snowfall (f=dndt, where dn is the new snow depth, and dt is the total snow depth (including new snowfall). ds is the densification due to overburden, which is parameterized as:

ds=ρi+SWEρmax-ρps     (40)

where SWE is the snow water equivalent, ρ is the snow density (after the addition of new snow) and  ps is a user-definable parameter specifying densification rate due to overburden.  dw is the densification due to wind effects, which is parameterized here as:

dw=wρmax-ρpw   (41)

where w is the wind speed, and pw is a user-definable parameter specifying densification rate due to wind.  dl is the densification due to liquid water in the pack, and is parameterized here as:

dl=rρmax-ρpl    (42)

where r is a flag specifying whether the snowpack is ripe (1 if ripe, and 0 if not ripe), and pl is a user-definable parameter specifying densification rate due to liquid in the snowpack.

2.11) Near Surface Hydrology Model

After the rainfall and snowmelt are combined, it is infiltrated into the soil.  Infiltration runoff is computed as

Ri=NWI×Fm×(max⁡(0,SM-SMi,min) /(SMi,max-SMi,min)   (43)

where NWI is the net water input, Fm is the maximum possible fraction of infiltration excess runoff (e.g. at saturated conditions), SM is the soil moisture content, SMi,max is the soil moisture at which infiltration excess runoff is maximized.

Next, evapotranspiration is computed as:

ET=PET×min⁡(1,max0,SM-WP/(CMC-WP)     (44)

where CMC and WP are, respectively, critical moisture content, and wilting point.  PET is a function of daily incoming solar radiation and air temperature as in Martel et al., 2017:

PET=Rs/(ρ×Lf)×(Tm+5)/100    (45)

where Rs is the extraterrestrial radiation in MJ/m2/day, ρ is the density of water (1000 kg/m3), Lf is the latent heat flux (2.26 MJ/kg), and Tm is the daily mean temperature.

Percolation out of the top soil layer is computed as:

D=k×SM-SMresSMsat-SMres2b+3  (46)

where k is the saturated hydraulic conductivity, SM is the soil moisture, SMsat is the saturated soil moisture, SMres is the residual soil moisture, and b is the pore size distribution index.  As in Kineros2, the soil specific residual soil moisture content is computed as an empirical function of the soil capillary drive parameter,

SMres=0.1+0.31+1g/100040.25   (47)

Capillary rise, itself, is computed as:

cr=bwkαψa/H2×b/3      (48)

where bw is SM-SMresSMsat-SMres, k is the saturated hydraulic conductivity, α=1+(3/2)/((2×b3)-1) ψa is the air entry pressure, H is the thickness of the upper soil layer, and b is the pore size distribution index.

Below the top soil layer, there are two conceptual layers representing 1) the rest of the vadose zone, and 2) the phreatic zone, which enables computation of baseflow originating from the deep soil layers.

Discharge from both reservoirs is computed as:

q=aSb      (49)

where S is the storage in each reservoir, and a and b, are constants.  There is also a leakage term between the two reservoirs, which is simply computed as

ql=cSvadose      (50)

where Svadose is the vadose zone storage, and c is a constant.

In addition to simulating water flow into and out of the top soil layer, RHEM-Snow also simulates heat conduction through this layer, as well as soil freezing / thawing.  Temperature is computed similarly to the snowpack, where energy can be added / removed from the soil layer from above or below:

Qsoil,t=Qsoil,t-1+Qtop+Qbottom      (51)

where Qtop and Qbottom are heat fluxes into and out of the top and bottom of the soil layer are computed using equation 29 (where the surface / snowpack temperature are used to compute the flux at the top, and a constant temperature and a damping depth are used to compute the flux at the bottom.

When the soil temperature is above or below freezing, the energy is used to warm and cool the soil temperature, where:

Tsoil=Qsoilρi×SMi×Ci+ρw×SMw×Cw+ρs×(H-SMi-SMw)×Cs   (52)

where Qsoil is the energy content of the soil layer, SMi is the mass of ice in the soil layer, SMw is the amount of liquid water in the soil layer, H is the soil layer thickness, Ci, Cw, and Cs are the specific heats of ice, liquid water, and soil, and ρw, ρw, and ρs are the densities of ice, water, and dry soil.

When the temperature drops to freezing or rises to freezing, the energy is used to drive soil melt / freezing.  This is calculated using equation 31, except Qm is the cold content of the soil layer, and the amount of ice produced is limited between 0 (unfrozen condition) and the soil moisture content (all water is frozen), and excess energy is used to raise and lower the soil temperature using equation 52.

3) Forcing Data

RHEM-Snow is designed to be forced with daily weather data available from the CLIGEN weather data generator.  As such, it requires a minimal number of forcing variables as input.  These are provided to the model as CLIGEN forcing data (.stm) files.  However, the model can also be run with historical forcing data (which are currently fed to RHEM-snow as forcing tables in Matlab binary (.mat) files.   Examples of these files can be found in Data/Cligen (*.stm), Data/ AZ_SNOTELs (*.mat), and Data/ReynoldsCreek (*.mat).

Required Forcing Inputs:

- Daily Precipitation – mm/day

- Daily Max, Min Temperature – degrees-C

- Daily Humidity (Dewpoint Temperature) – degrees-C

- Daily Incoming Solar (Shortwave) Radiation – langleys/day

- Daily Wind Speed – m/s

Optional Forcing Inputs:

- Daily Incoming Longwave Radiation  - langleys/day

- Daily Snowfall – mm/day

The CLIGEN files only contain the required forcing variables.  When run with CLIGEN files, the optional forcing inputs are estimated using the rainfall / snowfall partitioning and longwave schemes.  Each CLIGEN file contains data for one station, although multiple files can be given to RHEM-Snow for simultaneous execution for multiple sites (in which case, the timesteps must match).  As described in section 4, each station can also have different model parameters.

Mat files contain separate variables for daily precipitation (called prcp), maximum and minimum temperature (called tmax and tmin), dewpoint temperature (called tdpt), incoming solar radiation (called srad), wind speed (called wind), and optionally, snowfall (called snowfall) and longwave radiation (called lrad).  In addition, there are variables giving the start and end dates of the simulation (StartDate and EndDate).  Each of these variables is a n×m matrix where n is the number of simulation days and m is the number of stations to be simulated.

RHEM-Snow will automatically simulate all of the days and locations for which forcing data is provided, and so decisions about simulation length and location are made during the selection of the forcing data.  There is currently no functionality to stop or start simulations in the middle of the forcing timeseries.

4) Model parameters

There are 63 model parameters in RHEM-Snow, which can broadly be divided into eight categories: spatial parameters, initialized values, model forcing parameters, those related to the snow albedo model, those related to the snow density model, those related to the snow interception model, miscellaneous snowpack parameters, and hydrology model parameters.

Spatial Parameters (note that default values are defined in Preprocess.py, though they can be adjusted)

- latitude – for computing sun angles if either slope / aspect influences or cloudiness for incoming longwave radiation are calculated by RHEM-Snow [degrees]

- elevation - elevation

- slope – slope angle [degrees]

aspect – aspect [degrees from north, clockwise]

- lai – leaf area index (for computing canopy interception) [m2/m2]

Initialized Values:

- swe_i – Initial snow water equivalent [mm]

- swe_age_a_i – Initial effective age of snow surface since last snowfall [day]

- density_i – Initial snow density [g/cm3]

- cansnowstor_i – Initial canopy intercepted snow storage [mm]

- Tm_i – Initial snowpack temperature [C]

- T_soil_i – Initial soil layer temperature [C]

- ice_fraction_soil_i – Initial soil ice fraction [cm3/cm3]

- sm_i – Initial soil moisture [cm3/cm3]

- q_vadose_i – Initial water content in the vadose zone (below the top soil layer) [mm]

- q_phreatic_i - Initial water content in the preatic zone [mm]

Forcing Data Parameters:

- srad_mult – Shortwave Radiation Multiplier (simple correction for forcing bias, e.g. if simulation is for a different spatial position as forcing data)

- lrad_mult – Longwave Radiation Multiplier (simple correction for forcing bias, e.g. if simulation is for a different spatial position as forcing data)

- snow_mult – Snowfall Multiplier (simple correction for forcing bias, e.g. for things like snowfall undercatch)

- temp_adj - Adjustment to air temperature [C]

- RainThresh - Rain / Snow Transition Temperature [C] (at which rain and snow make up equal parts)

- RainThresh_dh - Range of temperatures over which the rain/snow transition occurs [C]

- CloudTransmission - Fraction of potential solar radiation to be considered cloudy (for calculating longwave radiation) [-]

  - e.g. a value of 0.4 would mean that if observed shortwave radiation is ≤ 0.4 that of potential shortwave radiation, then it would be considered to be cloudy

- use_tdew_ppm – Flag to specify whether to use dewpoint temperature instead of air temperature when computing the rain-snow transition (true = use dewpoint)

Snow Albedo Parameters:

These parameters relate to the time-varying snow albedo parameterization based on the age of the snow surface since the last significant snowfall

- albedo_snow_reset – Size of daily snowfall to reset the snowpack surface age [mm]

- albedo_decay - Snow albedo decay rate [fraction/day]

- albedo_i - Albedo of fresh snowpack [-]

- minalbedo - Minimum snowpack albedo [-]

Snow Density Parameters:

- density_i - Density of new snow [g/cm3]

- density_max - Maximum snow density [g/cm3]

- apar - Snow densification rate due to age [Fraction / day]

- dpar - Snow densfication rate due to overburdin [Fraction / cm [SWE]]

- rpar - Snow densfication rate due to liquid in snowpack [Fraction when isothermal snowpack]

Snow Interception Parameters

- melt_drip_par - Melt Drip Rate [mm/day per deg-C above freezing] snow_unload_par - Fraction of canopy snow that unloads each day [-]

- canopy_sub_mult - Canopy sublimation multiplier applied to potential sublimation rate [which is computed for snowpack surface] [-]

Miscellaneous Snowpack Parameters

- Ch - Multiplier applied to sensible heating equation

- ground_sub_mult - Ground sublimation multiplier applied to potential sublimation rate [-]

- sroughness - Snow surface roughness length [m]

- windlevel - Height of windspeed measurement [m]

- fstab – Stability parameter[-] (0-1; 1:  totally on Richardson number corrections; 0: assumes neutral atmospheric stability)

- kappa_snow - Snow thermal conductivity [W/m2/K]

- kappa_soil - Soil thermal conductivity [W/m2/K]

- tempdampdepth - Temperature at damping depth underneath a snowpack [C]

- dampdepth - Damping depth [m]

- albedo_0 - Albedo of snow-free ground

- groundveght- Ground Vegetation height [m]

Hydrology Model Parameters

- PET_Mult – Potential evapotranspirataion multiplier [-]

- max_infil_mult – Fraction of incoming water that becomes infiltration excess when the soil moisture is above the level specified by sm_max_infil [-]

- sm_min_infil – Soil moisture content below which infiltration excess runoff is minimized [-]

- sm_max_infil – Soil moisture content above which infiltration excess runoff is maximized [-]

- H – Thickness of surface soil layer [mm]

- ssat – Saturated water content [-]

- psi_s – Soil air entry pressure [cm]

- g – g Parameter for calculating residual soil moisture

- b_soil – Pore size distribution index [-]

- k_soil – Saturated hydraulic conductivity [mm/day]

- wp – Wilting Point [-]

- cmc – Critical Moisture Content [-]

- coef_vadose – Vadose Zone Reservoir Decay Parameter (multiplier) [-]

- coef_vadose_exp – Vadose Zone Reservoir Decay Parameter (exponent) [-]

- coef_vadose2phreatic – Leakage Rate between Vadose and Phreatic Reservoirs [-]

- coef_phreatic – Phreatic Zone Decay Parameter (multiplier) [-]

- coef_phreatic_exp – Phreatic Zone Decay Parameter (exponent) [-]

Model parameters can either be adjusted on a global or per-site basis.  If changing the global calibration, it is recommended to set them in the script that is running the model (e.g. RunRhemSnow_Testing.ipynb).  For example, to change the minimum snow albedo to 0.5, enter the line “modelpars[‘minalbedo’] = 0.5” right after “model_pars = Preprocess.default_model_pars(nlocs)”. If desired, the default model parameters can also be changed in preprocess.py (subroutine default_model_pars).

- To change the amount of snow accumulation adjust modelpars.RainThresh to change the partitioning between rain and snow or modelpars.snow_mult to apply a simple multiplier to snowfall

- modelpars.CloudTransmission affects the incoming longwave radiation under cloudy skies, though radiation adjustments can also be accomplished by changing modelpars.srad_mult and modelpars.lrad_mult.

- Changing the albedo parameters will have the greatest effect on snowmelt rate

- Changing the snow density parameters changes the relationship between snow depth and SWE (without much affecting other aspects of the model)

- The interception parameters only affect snow interception processes

- The miscellaneous snowpack parameters affect the computation of cold content, turbulent heat fluxes, and ground heat flux.  The most consequential of these parameters are modelpars.sroughness, which primarily affect latent heat flux and sublimation amounts: higher values of modelpars.sroughness will result in more sublimation.

- At this point, the soil model is still experimental.  Ultimately, it is designed to simulate near surface soil temperature and moisture content for initializing kineros2 runs.

- The spatial parameters as well as the forcing multipliers are designed to be given a priori (to account for spatial characteristics or known forcing deficiencies), but these can be changed for model testing, or as a way to account for site-specific characteristics such as shading or enhancement of longwave radiation by trees.  Note that interception is not calculated unless LAI is non-zero.  Also note that typically, at least latitude and elevation are given (e.g. they are read automatically from CLIGEN files).

5) Running RHEM-Snow

5.1) Input Data

The first step when running RHEM-Snow is to prepare the forcing data input for the simulation period, consisting of daily timeseries of at least precipitation, max/min temperatures, dewpoint temperature, wind speed, and solar radiation, and optionally snowfall and incoming longwave radiation.  The required variables are included in CLIGEN files and there is a built-in function to read these files (preprocess.py - subroutine get_forcing_cligen). Optional forcing inputs (snowfall, incoming longwave radiation) are not included in CLIGEN files.  These stations will be run with the default parameter set (defined in preprocess.py - subroutine default_model_pars) unless parameter values are modified when running the model (see RunRhemSnow_Testing.ipynb for examples of how to do this).

Defining custom forcing datasets is a little more involved, and since RHEM-Snow currently accepts forcing data in matlab .mat files (which are read in using scipy io functions), separate scripts must be used to generate these files from existing datasets (such as Data/ReynoldsCreek/prepare_data_sheltered.m).  These files must contain separate tables for all forcing variables (as described in section 3).

5.2) Model Execution

To execute RHEM-Snow, two commands need to be executed.  The first gets the forcing data and prepares it for the snow model, and the second runs the snow model.  Different forcing data scripts are called depending on whether CLIGEN files or .mat files are used (preprocess.py - subroutines get_forcing_cligen and get_forcing_mat).  Model.py (subroutine run_model) runs the snow model regardless of whether CLIGEN or .mat forcing data is used.  RunRhemSnow_Testing.ipynb give examples of how to run RHEM-Snow with both CLIGEN and .mat forcing data.

5.3) Model Output

Once RHEM-Snow has been run, model output can be saved by simply using scypy’s ‘savemat’ command.  For Cligen data, there is an additional script that will save input files for kineros (Output.py - function write_ts_output).   In addition, custom output tables can be generated from RHEMSnow outputs with a small amount of coding (RunRHEMSnow_InitialSites.ipnb gives an advanced example of how to perform these output functions.

There are also functions to display model balances and model results against validation data (Output.py - functions display_snowpack_validation and display_snowpack_balance).  For validation, it is recommended that the user supply their own validation script, but for convenience, the display_snowpack_validation function will plot timeseries of SWE, snow depth, snowfall, and sublimation against a validation timeseries (as well as its seasonal cycle).  To use this functionality, the validation data must be in a .mat file with separate tables for each variable, along with a vector of timesteps.  If using the display_snowpack_validation function to visualize validation results, possible validation variables are “swe”, “depth”, “snowfall”, and “sublimation”.  If these variables do not exist in the given .mat file (or if no .mat file is given), they will not be displayed against the model data.  Examples of how to use the output / display functionality are given in RunRhemSnow_Testing.ipynb.

6) Explanation of Model Codes

The Scripts directory contain all model codes (including auxiliary functions as well as those to import forcing data and write output files).  These scripts are not reproduced here, but below is a description of the purpose of each one.

Preprocess.py – contains functions to initialize the model parameters and get and process forcing datasets

function default_model_pars – sets default model parameters for all modeled locations.  Note that after default model parameters have been loaded, they can be changed manually.

The function is called with no inputs, and output from this function is a structure containing the default model parameter values.

function get_soil_pars – Function to populate soil model parameters based on soil type (uses Clapp and Hornberger, 1978 lookup table)

function get_forcing_cligen - Function to get cligen forcing data from one or more cligen files, prepare the data for RHEM-snow, and get their associated site-specific parameter values from parameter files (if any).

It is called with a list of cligen files as well as a structure containing default model parameters, which will be either applied to all of the sites or overwritten depending on site-specific parameter (.par) files are present for the cligen files.

Output from this function is a vector of python timestamps that correspond to the forcing data (note that if multiple forcing datasets are used together, the timestep information must match for all files), a structure containing this forcing data, formatted for use with RHEM-Snow, and the updated structure containing the model parameters for each site.

function get_forcing_mat - Function to get cligen observed forcing data from a .mat file, prepare the data for RHEM-snow, and get their associated site-specific parameter values from parameter files (if any).

It is called with the name of an input .mat file, a structure containing default model parameters, which will be either applied to all of the sites or overwritten depending on site-specific parameter files are present in the .mat file, and 2 flags to use pre-partitioned precipitation data in the forcing data files, as well as to use given (from the forcing data) or calculated (using subroutines in RHEM-Snow) longwave radiation values.

Output from this function is a vector of python timestamps that correspond to the forcing data (note that if multiple forcing datasets are used together, the timestep information must match for all files), a structure containing this forcing data, formatted for use with RHEM-Snow, and the updated structure containing the model parameters for each site.

function solarradiation – Function to calculate solar radiation for a digital elevation model (DEM) for all days of the year for clear sky conditions following the approach of Kumar, 1997.

This script is called by the get_forcing functions, and so does not need to be called by users of RHEM-Snow.

It is a slightly modified version of a Matlab script on the Matlab file exchange by Felix Hebeler.  Its modifications are relatively minor, allowing it to accept values of slope and aspect used for RHEM-Snow, separate the solar radiation values (which were aggregated for the entire year) into daily values, and correctly populate the structures for RHEM-Snow modeled locations.

Model.py – contains functions to run RHEM-Snow and disaggregate model outputs

function run_model – Function to run RHEM-Snow

This function is called with the output data structure from the get-forcing-data functions (a vector of python timestamps for the forcing data as well as structure containing the forcing data and the model parameters.

Output from this function is a structure containing all model outputs for all locations where the model is run.  Note that each field will match the size of each of the forcing data fields where rows correspond to the day of simulation, and columns correspond to the location that is simulated).

function get_ts_data – Function to disaggregate net water input from RHEM-Snow

This function is called with the output data structure from the get-forcing-data and run_model functions, as a scalar indicating the desired timestep (fraction of a day)

Output from this function are disaggregated timeseries of rainfall and melt

-------------------------------------------------------------------------------------------------------------------------------

Output.py – contains functions to write out and display model output

function write_ts_output - Function to write diaggregated model output from RHEM-Snow to Kineros input files

The function needs timestamp information, disaggregated net water input timeseries, and ice and saturation values for each day

This function writes a directory of input files for kineros simulations

Function display_snowpack_validation - Function to display model validation of a RHEM-Snow simulation

The function needs to be called with the forcing_data and model_output structures, the name of a file containing validation data, a vector of timestamps, and a range of dates to display data for.

Function display_snowpack_balance - Function to display mass and energy balances from a Rhem-Snow simulation.

The function needs to be called with the forcing_data and model_output structures, a vector of timestamps, and a range of dates to display data for.

7) References

Anderson, E. A., (1976), "A Point Energy and Mass Balance Model of a Snow Cover," NOAATechnical report NWS 19, U.S. Department of Commerce.

Bristow,  K.  L.  and  G.  S.  Campbell,  (1984),  "On  the  Relationship  Between  Incoming  SolarRadiation  and  the  Daily  Maximum  and   Minimum   Temperature,"   Agricultural   and   ForestMeteorology, 31: 159-166.

Brutsaert, W., (1975), "On a deriveable formula for long-wave radiation from clear skies," WaterResources Research, 11: 742-744.

Hedstrom, N. R., and J. W. Pomeroy. "Measurements and modelling of snow interception in the boreal forest." Hydrological Processes 12, no. 10‐11 (1998): 1611-1625.

Kumar, L, Skidmore AK and Knowles E 1997: Modelling topographic variation in solar radiation in a GIS environment. Int.J.Geogr.Info.Sys. 11(5), 475-497

Liston, Glen E., and Kelly Elder. "A distributed snow-evolution modeling system (SnowModel)." Journal of Hydrometeorology 7, no. 6 (2006): 1259-1276.

Male, D. H. and D. M. Gray, (1981), "Snowcover Ablation and Runoff," Chapter 9 in Handbookof  Snow,  Principles,  Processes,  Management  and  Use,  Edited  by  D.  M.  Gray  and  D.  H.  Male,Pergammon Press, p.360-436.

Martel, J-L, Demeester, K., Brissette, F., Poulin, A., Arsenault, R., HMETS—A simple and efficient hydrology model for teaching hydrological modelling, flow forecasting and climate change impacts." International Journal of Engineering Education Vol. 33, No. 4, pp. 1307–1316, 2017

Monteith, J.L., and Unsworth, M.H. 2008. Principles of Environmental Physics. Third Ed. AP, Amsterdam. http://store.elsevier.com/Principles-of-Environmental-Physics/John-Monteith/isbn-9780080924793/

Pomeroy, J. W., J. Parviainen, N. Hedstrom, and D. M. Gray. "Coupled modelling of forest snow interception and sublimation." Hydrological processes 12, no. 15 (1998): 2317-2337.

Price,  A.  G.  and  T.  Dunne,  (1976),  "Energy  balance  computations  of  snowmelt  in  a  subarcticarea," Water Resources Research, 12(4): 686-694.

Satterlund, D. R., (1979), "An Improved Equation for Estimating Long-wave Radiation From theAtmosphere," Water Resources Research, 15: 1643-1650.

Tarboton, David G., and Charles H. Luce. Utah energy balance snow accumulation and melt model (UEB). Utah Water Research Laboratory, 1996.